NEW ZEALAND BUILDING CODE DOCUMENTATION FOR FLOOR COVERINGS (LVT)

Consent detail: CONSENT APPLICATION NUMBER / ADDRESS HERE

Floorscape Product: LVT - ENTER BRAND AND STYLE NAME HERE

COUNCIL NAME AND ADDRESS DETAIL HERE

Dear NAME,

Scope: Provide required documentation to provide evidence to satisfy the relevant clauses set out in Schedule 1 of New Zealand Building Code for the nominated Floor Covering in both single-level and multi-level buildings.

Composition of Floorscape LVT product:

The New Zealand Building Code clauses and evidence relevant to this consent for Floorscape product are listed below:

| NZBC Code Clause | Compliance Information Used | How the information is used | Evidence |
| --- | --- | --- | --- |
| SINGLE-LEVEL E3-Internal Moisture E3.3.3; E3.3.5 Re: Impervious and Easily cleaned surfaces B2-Durability B2.3.1 (c) 5 years (i) Re: linings to satisfy the performance requirements with normal maintenance where they are easy to access and replace. (ii) Re: failure is easily detected during normal use of the building. ##### F2.- Hazardous building materials F2.3.1 Re: harmful emissions from materials used. (Formaldehyde is a naturally occurring chemical in wood and is commonly used in the production process of wood composite panels E.g. Cabinetry, Furniture, Plywood, Chipboard etc.) MULTI-LEVEL G6- Airborne and impact sound G6 3.1 Sound Transmission no less than 55 G6 3.2 Impact Insulation no less than 55 C1- Objectives of clauses C3 Fire affecting areas beyond the fire source Re: multi-story dwelling – common areas D1- Access routes D1.3.3-Re: have adequate slip-resistance walking surfaces under all conditions of normal use | ISO4760 Topical Moisture Resistance – Assembles Joint Manufacturer’s specifications Care and Maintenance instructions EN16511 Standard: Loose-laid panels – Semi-rigid multilayer modular floor covering (MMF) panels with wear resistant top layer EN717-1 VOC Emission Standard: Wood-based Panels – Determination of Formaldehyde Release – Formaldehyde emission by the chamber method. ISO140-7 Sound Transmission and Insulation standard: Acoustics — Rating of sound insulation in buildings and of building elements ISO9239-1 Reaction to Fire AS4586:2013 Slip Resistance | Independently tested in New Zealand by New Zealand Wool Testing Authority to demonstrate over a period of 24 hours that the sample joints tested did not allow water penetration through to the substrate. Three boards are assembled together, and a 100mm diameter plastic cylinder is sealed to the board directly above the T-Joint. 100mm of water is then placed into the cylinder with 5 drops of colour dye. This is left for 24 hours, after which the cylinder is removed and the joint disassembled, to expose what if any moisture had passed through the board. This demonstrates that the surface of the board (the LVT and the board joints did not allow moisture penetration and can therefore be considered impervious). Floorscape Care and Maintenance instructions to demonstrate that Floorscape product is Impervious and Easy to clean Independent Taber testing for classification requirements to EN16511 AC rating per Table 2 of EN16511 standard Normal maintenance requirements for these areas are set out in the Floorscape Care and Maintenance instructions to cover ‘water splash’ per E3.3.3 classification within NZBC. This product is not suitable for inside a shower cubicle (continuous wet area) Independent testing to demonstrate E1 result which is designated under the lowest category for formaldehyde emission per EM717-1 standard (Release ≤ 0.124 mg/m3 air). Sound Insulation is nominated in the NZBC clause G6 and requires an STC and IIS rating of no less than 55. We have independently tested to demonstrate that Sound Transmission Class and Impact Insulation Class shall be no less than 55 through ISO140-7 testing. Note: ISO140-7 Test Method encompasses both Impact & Transmission Sound: These ratings are then combined and divided by 2 to provide an average which per NZBC can be no less than 55. In the certificate attached we have an L’nT value of 53 and IIC value of 54, added = 107. Divided by 2 = 53.5. ISO9239-1 is nominated in the NZBC: clause C3 Fire affecting areas beyond the fire source The attached independent test report confirms this this requirement has been achieved AS4586 test is nominated in the acceptable solution clause D1.3.3 (d) The attached independent test report confirms this requirement has been achieved | NZWTA Certification Floorscape Care and Maintenance instructions to be followed by consumer NZWTA Taber Test Certification Eurofins VOC test report SLR ISO140-7 Certification AWTA Certification ATTAR Certification |

Based on the above, and the certification that is provided attached to this document, we believe that this product is fit for purpose and meets the relevant clauses set out in Schedule 1 of the New Zealand Building Code.

Installation instructions and Technical Data Sheets can be provided on request.

Thanks and Regards

YOUR NAME AND SIGNATURE